Intellis Solar Power Generation Scheduling System
Day-Ahead (DA) Low-Level Algorithmic Specification, Multi-Agency Selection Engine, and LLM JSON-to-CSV Pipeline

System Version: 3.4.0-Solar-DA ¦ Author: Vedanjay Power Intellis Architecture Team ¦ Target Domain: Day-Ahead Solar Power Forecasting (96 Blocks) ¦ Document Type: Low-Level Algorithmic Specification

1. Global Variables, System Parameters & Plant Profile Usage Matrix

This section documents the operational parameters, mathematical constants, 24h-48h forecast lead-time thresholds, diurnal MOS selection parameters, clear-sky irradiance bounds, and statutory gate-closure timing governing the Day-Ahead (DA) Solar Generation Forecasting Engine. In contrast to intraday rolling revisions, Day-Ahead forecasting operates without live telemetry for Day T+1 and relies on multi-model NWP super-ensembles, physical transposition, 24h lead-time historical MOS calibration, and strategic LLM regime arbitration.

| Parameter Name | Symbol / Variable | Default Value / Bound | System Description & Role |
| --- | --- | --- | --- |
| AC Interconnect Capacity | P_CAP_AC | 5.0 - 50.0 MW | Rated active power export capacity at grid interconnection point. |
| DC Installed Capacity | P_CAP_DC | 6.5 - 65.0 MWp | Total installed peak DC capacity of solar PV array under STC. |
| DC-to-AC Transfer Ratio | eta_transfer | 0.75 - 0.85 | Physical transfer ratio converting DC array output to AC export power. |
| Site Barometric Altitude | z | 250.0 - 650.0 m | Site elevation above mean sea level used for barometric air pressure calculation. |
| Panel Tilt Angle | beta | 15.0° - 24.0° | Fixed tilt angle of solar PV module array relative to horizontal plane. |
| Panel Azimuth Angle | gamma_m | 180.0° (South) | Orientation angle of PV array relative to True North (180° = South facing). |
| Linke Turbidity Factor | T_L | 3.0 - 4.5 | Atmospheric aerosol and water vapor attenuation factor in Ineichen clear-sky model. |
| Temperature Coefficient | gamma_Pmp | -0.37% / °C | Temperature derating coefficient of maximum power for PV modules. |
| Faiman Convection Coeff | u0, u1 | u0=29.0, u1=0.0 | Sandia Faiman thermal model constants converting GTI and wind speed to cell temperature. |
| DA Gate Closure Time | T_gate_closure | 10:00 AM / 12:00 PM IST | Statutory submission cut-off time on Day T for Day T+1 grid schedule. |
| DA Horizon Blocks | H_da | 96 blocks (24 Hours) | Total 15-minute grid blocks optimized for Day T+1 (00:00 to 24:00 IST). |
| Forecast Lead Time | L_lead_time | 24h - 48h Lead | Temporal lookahead window separating Day T submission from Day T+1 generation. |
| NWP Candidate Pool | N_nwp_members | 143 Members | Total multi-agency ensemble model candidates (ECMWF, ICON, GFS, GEM). |
| Agency Diversity Cap | C_agency_cap | Max 2 / Agency | Maximum candidates selected per NWP family in a diurnal slot to enforce diversity. |
| Slot Champion Count | K_slot_champions | 6 Models / Slot | Top ranked candidate models selected for Bayesian inverse-loss weighting. |
| LLM Sampling Temperature | T_llm | 0.20 | Sampling temperature for OpenRouter GPT-5.6 Luna to enforce deterministic reasoning. |
| LLM Foundation Model | Model_LLM | openai/gpt-5.6-luna | Strategic LLM arbiter model invoked via OpenRouter API Gateway for DA regime positioning. |
| API Retry Pool Size | N_key_retries | 3 API Keys | Resilient multi-key rotation pool with exponential backoff on HTTP 429 / 503. |

1.1 Plant Profile Information Usage Matrix across Intermediary Steps & LLM Prompts

To ensure complete transparency and prevent illegal information leakage, the table below specifies exactly which plant profile attributes are consumed during intermediary computational physics steps versus which attributes are injected into the Master LLM Prompt:

| Plant Profile Attribute | Intermediary Computation Step Usage | LLM Prompt Injection Usage | System Role & Purpose |
| --- | --- | --- | --- |
| Plant Name & Site ID | File naming & S3 data path resolution | Injected in Prompt Header | Identifies target asset for logging and prompt context. |
| Latitude & Longitude | Module 2: Solar zenith & azimuth ephemeris | Excluded from LLM prompt | Used exclusively for NREL SPA astronomical ephemeris. |
| Barometric Altitude (z) | Module 2: Air mass & Ineichen barometric pressure | Excluded from LLM prompt | Scales atmospheric path length for direct clear-sky irradiance. |
| Fixed Array Tilt (beta) | Module 2: Perez / Hay-Davies POA transposition | Excluded from LLM prompt | Transforms horizontal GHI to plane-of-array GTI. |
| Array Azimuth (gamma_m) | Module 2: Angle of incidence (AOI) calculation | Excluded from LLM prompt | Determines sun-facing panel orientation relative to South. |
| DC Installed Capacity (P_DC) | Module 4: DC array power simulation | Injected in SECTION 1 System Constraints | Defines total DC STC nameplate rating for array output. |
| AC Interconnect Capacity (P_AC) | Module 4, 6, 9: Inverter clipping & guardrails | Injected in SECTION 1 System Constraints | Hard upper bound for active grid power export. |
| DC/AC Transfer Ratio | Module 4: Inverter conversion efficiency | Injected in SECTION 1 System Constraints | DC-to-AC conversion scaling factor (P_AC / P_DC). |
| Monthly Linke Turbidity (T_L) | Module 2: Ineichen clear-sky radiation attenuation | Excluded from LLM prompt | Quantifies aerosol and water vapor clear-sky absorption. |
| Inverter Outage Schedule | Module 6: Active capacity envelope P_avail(b) | Injected as Active Capacity Ceiling | Derates export capacity based on hardware maintenance. |

2. End-to-End System Execution Flow & Dynamic IST Date Resolution

Whenever the Day-Ahead Solar Generation Forecasting Engine is executed for any plant at any timestamp, it dynamically resolves the current local date in timezone Asia/Kolkata (D_today). Based on D_today, it sets the target forecast date strictly to D_target = D_today + 1 day (Day T+1, 96 blocks from 00:00 to 24:00 IST) and fetches weather ensemble feeds specifically for D_target. The complete 10-step sequential low-level algorithmic pipeline is detailed below:

Step 1: Dynamic IST Execution & Target Date Resolution (08:30 AM IST Gate): Resolve system execution timestamp in timezone Asia/Kolkata: T_now = datetime.now(timezone('Asia/Kolkata')). Set D_today = T_now.date(). Automatically compute target Day-Ahead forecast date: D_target = D_today + timedelta(days=1). Ingest plant hardware profile parameters. Query Open-Meteo High-Bandwidth API for target date D_target specifically; download 00Z/06Z runs for all 143 ensemble candidates (m = 1..143) for hourly vectors [GHI_m(H), DNI_m(H), DHI_m(H), T_air_m(H), wind_10m_m(H), cloud_cover_m(H)] for H in [0..23]. Query AWS S3 data lake path s3://{S3_BUCKET}/raw/vedanjay/{site_id}/scada_actuals/ to retrieve 7-day historical SCADA actual generation matrix A(d, b) for past days d in [D_today-7 .. D_today-1] and b in [1..96].

Step 2: NREL SPA Solar Position Ephemeris & Ineichen Clear-Sky POA Transposition: FOR each 15-minute midpoint block b in [1..96] of target date D_target (timestamp t_b): evaluate solar zenith angle theta_z(b), solar azimuth gamma_s(b), and solar elevation alpha(b) = 90° - theta_z(b) via NREL Solar Position Algorithm (SPA). Calculate relative air mass AM(b) and barometric altitude correction AM_corr(b) = AM(b) * (P_baro(z) / 101325). Compute Ineichen Clear-Sky Irradiance Triplet: DNI_cs(b) = b0 * I0 * exp(-0.09 * AM_corr * (T_L - 1)), GHI_cs(b) = DNI_cs(b) * cos(theta_z) + DHI_cs(b). Compute Angle of Incidence cos(theta(b)) = cos(theta_z)*cos(beta) + sin(theta_z)*sin(beta)*cos(gamma_s - gamma_m). Compute Hay-Davies / Perez clear-sky POA irradiance: POA_cs(b) = POA_beam(b) + POA_ground(b) + POA_diffuse(b). IF alpha(b) < 3.0° or theta_z(b) >= 87.0°: clamp POA_cs(b) = 0.00 W/m² strictly.

Step 3: Multi-Agency NWP Member Ingestion & Physics-Guided Kt Spline Reconstruction: FOR each candidate member m in [1..143]: FOR each hour H in [0..23]: compute hourly clearness ratio Kt_m(H) = clip(GHI_m(H) / max(1.0, GHI_cs(H)), 0.0, 1.20). Fit natural cubic spline S_m(t) over 24 hourly points {Kt_m(H)} with natural end conditions S''_m(0) = S''_m(23) = 0. Evaluate spline at 96 15-minute midpoints: Kt_m(b) = S_m(t_b) for b in [1..96]. Reconstruct high-resolution Plane-of-Array Irradiance matrix: GTI_m(b) = Kt_m(b) * POA_cs(b). IF b < 24 (Time < 05:45 IST) OR b > 74 (Time >= 18:30 IST): set GTI_m(b) = 0.00 W/m².

Step 4: Faiman/Sandia Thermal Convection Derating & AC Base Power Simulation: FOR each member m in [1..143] and block b in [1..96]: evaluate Sandia module cell temperature T_cell_m(b) = T_air_m(b) + GTI_m(b) * exp(-29.0 - 0.0 * wind_10m_m(b)). Compute thermal power derate factor: eta_temp_m(b) = 1.0 - 0.0037 * (T_cell_m(b) - 25.0). Calculate raw DC generation: P_dc_m(b) = P_CAP_DC * (GTI_m(b) / 1000.0) * eta_temp_m(b). Convert to AC export power: P_ac_m(b) = min(P_dc_m(b) * eta_transfer, P_CAP_AC). Apply 96-block site array transfer weights: P_base_m(b) = round(P_ac_m(b) * M[b], 2). Enforce clear-sky envelope ceiling: P_base_m(b) = min(P_base_m(b), P_clearsky(b)).

Step 5: 24h Lead-Time MOS Error Accounting & Top-6 Member Selection with Mandatory Agency Diversity Cap: FOR each candidate member m in [1..143] and block b in [1..96]: compute Mean Absolute Error on 24h-48h lead times across past 7 days: MAE_m(b) = (1/7) * SUM(|P_base_history(d, m, b) - A(d, b)|). Partition Day T+1 into 3 diurnal slots: Slot 1 (Morning: b in [24..40]), Slot 2 (Midday: b in [41..56]), Slot 3 (Evening: b in [57..74]). FOR each slot k in {1, 2, 3}: calculate slot average MAE MAE_bar_m,k = (1 / |B_k|) * SUM(MAE_m(b)). Rank members by ascending MAE_bar_m,k. Select top K=6 champions while enforcing MANDATORY AGENCY DIVERSITY CAP: max 2 models per NWP family (ECMWF 0..34, ICON 35..74, GFS 75..104, GEM 105..142). Why? Prevents single-model family bias from dominating the ensemble. If 6 members cannot be filled under 2-per-agency limit, relax cap to 3.

Step 6: Ensemble Aggregation Strategy (Bayesian Inverse-Loss Weighting vs Simple Mean): Simple average/mean is strictly rejected because equal-weighting treats an inaccurate member (MAE = 1.2 MW) identically to a high-accuracy member (MAE = 0.3 MW). Enforce Bayesian Inverse-Loss Weighting (Weighted Percentage): for top 6 selected slot champions i in [1..6], compute weight w_i,k = (1 / MAE_bar_i,k) / SUM(1 / MAE_bar_j,k). Apply diurnal MOS linear calibration gain P_calib_k(b) = a_k * SUM(w_i,k * P_base_i(b)) + b_k. Assemble 96-block profile using 4-block raised-cosine transition fading at slot boundaries (b in [39..42] and b in [55..58]) to yield P_mos_da[1..96].

Step 7: Planned Maintenance & Hardware Availability Derate Engine: Ingest scheduled maintenance registry for target date D_target: extract count of available inverters N_inv_avail(b) and total inverters N_inv_total. Compute active interconnect capacity envelope P_avail(b) = P_CAP_AC * (N_inv_avail(b) / N_inv_total). Apply capacity derate: P_mos_derated(b) = min(P_mos_da(b), P_avail(b)) for b in [1..96].

Step 8: Master Day-Ahead Prompt Construction & Atmospheric Context Assembly: Extract synoptic atmospheric indicators for D_target: mean convective CAPE (J/kg), cloud cover %, relative humidity %, AOD, NWP divergence. Build Section 1 (System Constraints & Statutory 10:00 AM Gate Closure rules), Section 2 (Synoptic Weather Indicators), Section 3 (24h MOS Portfolio Rankings), Section 4 (96-Block Comparison Data Matrix Table), Section 5 (Strict JSON Target Schema). Save prompt string to master_da_prompt_{site_id}_{D_target}.txt.

Step 9: Strategic OpenRouter LLM Arbitration (`openai/gpt-5.6-luna`): Configure OpenRouter HTTPS POST gateway endpoint https://openrouter.ai/api/v1/chat/completions (model = 'openai/gpt-5.6-luna', temperature = 0.20, max_tokens = 2500, response_format = {'type': 'json_object'}). Execute request with 3-key rotation pool [KEY_PRIMARY, KEY_SECONDARY, KEY_GLOBAL_FAILOVER]. On HTTP 429/503 error, apply exponential backoff retry (1s, 2s, 4s) and rotate to next key. Parse JSON payload: extract synoptic_regime ('CLEAR_SKY', 'HIGH_CIRRUS', 'MONSOON_OVERCAST', 'LOCAL_CONVECTIVE'), quantile_bias_factor in [0.70, 1.08], and block_predictions dictionary containing 96 values. IF key pool is exhausted or JSON parse fails: trigger 0ms fail-safe fallback path and set raw prediction vector P_raw_llm = P_mos_derated.

Step 10: LLM JSON-to-CSV Conversion, Physical Guardrails & Statutory Gate Dispatch (09:45 AM IST): Strip markdown fences (```json ... ```) if returned. Filter raw predictions through Guardrail 1 (Solar geometry night zeroing b < 24 or b > 74 => 0.00 MW), Guardrail 2 (Clear-sky floor lock), Guardrail 3 (Overcast optical cloud ceiling), and Guardrail 4 (Ramp continuity Delta_MW <= 10% P_CAP_AC & capacity clipping to P_avail). Format 96-block schedule into canonical 7-column DA CSV schema: [Block, Time Interval, clearsky_poa_w_m2, mos_consensus_mw, llm_quantile_mw, da_schedule_mw, active_capacity_mw]. Generate interactive Plotly HTML graphic da_evaluation_{site_id}_{D_target}.html. Upload all artifacts to s3://{S3_BUCKET}/raw/vedanjay/{site_id}/{D_target}/day_ahead/ via boto3 S3 client before 10:00 AM statutory gate closure. RETURN {DA_CSV_Path, Plotly_HTML_Path, S3_URI_List}.

3. Module-by-Module Algorithmic Logic

This section details the exact low-level algorithmic logic, mathematical equations, candidate selection logic, data input structures, output specifications, and step-by-step procedures for each of the 10 computational modules comprising the Day-Ahead Solar System.

3.1 Module 1: Dynamic Execution Resolution, Ingestion & Plant Configuration

• Algorithm Inputs: site_id, execution_timestamp (IST), P_CAP_AC, P_CAP_DC, beta, gamma_m, altitude z, S3_BUCKET.

• Algorithm Outputs: D_today, D_target, Site_Config struct, NWP_Raw_Feed[143] for D_target, Historical_SCADA_7D[7][96].

Algorithmic Procedure:

Step 1.1 Dynamic Date Resolution in Asia/Kolkata Timezone: Resolve execution timestamp: T_now = datetime.now(timezone('Asia/Kolkata')). Set D_today = T_now.date(). Calculate target forecast date: D_target = D_today + timedelta(days=1). Format target_date_str = D_target.strftime('%Y-%m-%d').

Step 1.2 Ingest Plant Hardware Configuration: Load plant capacity specs: P_CAP_AC (MW), P_CAP_DC (MWp), array tilt beta (deg), azimuth gamma_m (deg), barometric altitude z (m), Linke turbidity monthly array T_L.

Step 1.3 Synchronize Fresh 00Z / 06Z NWP Model Runs for Target Date: Query Open-Meteo High-Bandwidth API for target date D_target specifically. Download 00Z/06Z runs for all 143 ensemble candidates (ECMWF IFS 35 members, ICON-EU/Global 40 members, GFS/GEFS 30 members, GEM 38 members). Extract hourly GHI_m(H), DNI_m(H), DHI_m(H), T_air_m(H), wind_10m_m(H), cloud_cover_m(H) for target date D_target.

Step 1.4 Ingest Historical SCADA for 24h Lead-Time MOS Evaluation: Query AWS S3 path s3://{S3_BUCKET}/raw/vedanjay/{site_id}/scada_actuals/ for past 7 days (Days D_today-7 to D_today-1). Parse timestamps to Asia/Kolkata timezone. Exclude non-numeric #N/A values from error accounting.

Step 1.5 Sanity Check & Persistence: IF NWP feed contains missing timesteps: trigger linear forward-fill across hours. Save raw ingested feeds to da_raw_nwp_feed_{site_id}_{D_target}.json. RETURN {D_today, D_target, Site_Config, NWP_Raw_Feed, Historical_SCADA_7D}.

3.2 Module 2: NREL SPA / Ineichen Clear-Sky Solar Ephemeris & Transposition Engine

• Algorithm Inputs: latitude, longitude, altitude z, tilt beta, azimuth gamma_m, Linke turbidity T_L, D_target (Day T+1).

• Algorithm Outputs: POA_cs[1..96] (Deterministic Clear-Sky Plane-of-Array Irradiance array in W/m²).

Algorithmic Procedure:

Step 2.1 Astronomical Solar Position Calculation: FOR each 15-minute block midpoint t_b (b = 1..96) of target date D_target: evaluate solar zenith angle theta_z(b), solar azimuth angle gamma_s(b), and solar elevation angle alpha(b) = 90° - theta_z(b) using NREL SPA algorithm.

Step 2.2 Ineichen Clear-Sky Radiation Triplet Evaluation: Calculate relative air mass AM(b) with barometric altitude correction AM_corr = AM * (P_baro / 101325). Evaluate Ineichen Clear-Sky Triplet: Direct Normal DNI_cs(b), Diffuse Horizontal DHI_cs(b), Global Horizontal GHI_cs(b) = DNI_cs(b) * cos(theta_z(b)) + DHI_cs(b).

Step 2.3 Compute Angle of Incidence (AOI): Calculate AOI theta(b) on tilted PV array: cos(theta) = cos(theta_z) * cos(beta) + sin(theta_z) * sin(beta) * cos(gamma_s - gamma_m).

Step 2.4 Hay-Davies / Perez Plane-of-Array Transposition: Decompose clear-sky POA into 3 components: Direct Beam POA_beam = DNI_cs * max(0, cos(theta)); Ground Albedo Reflection POA_ground = GHI_cs * 0.20 * (1 - cos(beta)) / 2; Sky Diffuse POA_diffuse via Hay-Davies anisotropic model. Total POA_cs(b) = POA_beam + POA_ground + POA_diffuse.

Step 2.5 Low-Elevation Zenith Cutoff: IF solar elevation alpha(b) < 3.0° (or theta_z(b) >= 87.0°): clamp POA_cs(b) = 0.00 W/m² strictly.

Step 2.6 Persistence: Save theoretical envelope to da_clearsky_poa_{site_id}_{D_target}.csv. RETURN POA_cs[1..96].

3.3 Module 3: 143-Member NWP Super-Ensemble Fetch & Physics-Guided Kt Spline Engine

• Algorithm Inputs: NWP_Raw_Feed[143], POA_cs[1..96], GHI_cs[1..96], D_target.

• Algorithm Outputs: GTI_matrix_da[143][96] (Discretized 15-minute Plane-of-Array Irradiance matrix for Day T+1).

Algorithmic Procedure:

Step 3.1 Address Linear Interpolation Step Errors: Direct linear interpolation of hourly NWP irradiance feed creates severe unphysical trapezoidal step discontinuities near sunrise and sunset. Enforce Physics-Guided Clearness Ratio Kt Spline Reconstruction.

Step 3.2 Compute Hourly Clearness Ratio Kt_m(H): FOR each member m in {1, ..., 143} and hour H in {0, ..., 23}: compute hourly clearness index Kt_m(H) = GHI_m(H) / max(1.0, GHI_cs(H)). Clamp Kt_m(H) in [0.0, 1.20].

Step 3.3 Natural Cubic Spline Interpolation of Kt: Fit natural cubic spline over 24 hourly values Kt_m(H); evaluate spline at 96 15-minute block midpoints to produce continuous clearness profile Kt_m(b) for b in {1, ..., 96}.

Step 3.4 Physics-Guided High-Resolution GTI Reconstruction: Multiply continuous clearness profile by high-resolution instantaneous clear-sky POA: GTI_m(b) = Kt_m(b) * POA_cs(b). This perfectly recovers astronomical Earth rotation curvature while preserving NWP cloud attenuation.

Step 3.5 Matrix Validation: FOR each member m and block b: IF GTI_m(b) < 0.0: set GTI_m(b) = 0.0. IF block b < 24 OR block b > 74: set GTI_m(b) = 0.0.

Step 3.6 Persistence: Save matrix to da_gti_matrix_{site_id}_{D_target}.csv. RETURN GTI_matrix_da[143][96].

3.4 Module 4: Faiman/Sandia Convective Thermal Derating & Inverter Conversion Engine

• Algorithm Inputs: GTI_matrix_da[143][96], T_air_m(b), wind_10m_m(b), P_CAP_DC, P_CAP_AC, gamma_Pmp, u0, u1, M[b].

• Algorithm Outputs: P_base_matrix_da[143][96] (Thermally derated AC base power matrix for all 143 candidates).

Algorithmic Procedure:

Step 4.1 Evaluate Sandia Module Cell Temperature: FOR each member m and block b: evaluate PV cell temperature using Sandia Faiman model: T_cell_m(b) = T_air_m(b) + GTI_m(b) * exp(-u0 - u1 * wind_10m_m(b)) where u0=29.0, u1=0.0.

Step 4.2 Thermal Power Derating Calculation: Compute module temperature efficiency factor: derate_temp_m(b) = 1.0 + (gamma_Pmp / 100.0) * (T_cell_m(b) - 25.0) where gamma_Pmp = -0.37% / °C.

Step 4.3 DC Generation & AC Inverter Conversion: Compute raw DC power: P_dc_m(b) = P_CAP_DC * (GTI_m(b) / 1000.0) * derate_temp_m(b). Convert to AC export power via inverter efficiency curve: P_ac_m(b) = min(P_dc_m(b) * eta_transfer, P_CAP_AC).

Step 4.4 Ingest Site Empirical Transfer Multipliers: Multiply AC output by 96-block empirical site array transfer weights: P_base_m(b) = round(P_ac_m(b) * M[b], 2).

Step 4.5 Clear-Sky Envelope Clamping: Clamp base power strictly below theoretical clear-sky envelope: P_base_m(b) = min(P_base_m(b), P_clearsky(b)).

Step 4.6 Persistence: Save matrix to da_physics_base_power_{site_id}_{D_target}.csv. RETURN P_base_matrix_da[143][96].

3.5 Module 5: 24h-Lead-Time MOS Error Accounting & Top-6 Selection with Agency Diversity Cap

• Algorithm Inputs: P_base_matrix_history[7][143][96], Historical_SCADA_7D[7][96], P_base_matrix_da[143][96].

• Algorithm Outputs: P_mos_da[1..96] (Fused Day-Ahead MOS consensus power forecast vector).

Algorithmic Procedure:

Step 5.1 Evaluate 24h Lead-Time Historical Error: FOR each member m in {1, ..., 143} and block b in {1, ..., 96}: compute Mean Absolute Error (MAE) over rolling 7-day lookback specifically evaluated at 24h-48h forecast lead times: MAE_m(b) = mean(|P_base_history(d, m, b) - SCADA_history(d, b)|).

Step 5.2 Partition Day T+1 into 3 Diurnal Slots: Divide 96 blocks into 3 operational slots: Slot 1 (Morning: Blocks 24-40 / 06:00-10:00 IST), Slot 2 (Midday: Blocks 41-56 / 10:00-14:00 IST), Slot 3 (Evening: Blocks 57-74 / 14:00-18:30 IST).

Step 5.3 Enforce Top-6 Selection & Mandatory Agency Diversity Capping Algorithm: Rank candidates in each slot by ascending MAE. Select top K=6 champions while enforcing MANDATORY AGENCY DIVERSITY CAP: max 2 members per NWP family (ECMWF 0..34, ICON 35..74, GFS 75..104, GEM 105..142). Why? Prevents single-model family bias from dominating the ensemble. Pseudocode: Initialize selected = [], agency_counts = {ECMWF:0, ICON:0, GFS:0, GEM:0}. FOR m in ranked_indices: agency = get_agency(m). IF agency_counts[agency] < 2: append m to selected; agency_counts[agency] += 1; IF len(selected) == 6: break.

Step 5.4 Compute Bayesian Inverse-Loss Weights (Weighted Percentage vs Mean): Simple mean is strictly rejected because equal-weighting treats an inaccurate member (MAE = 1.2 MW) identically to a high-accuracy member (MAE = 0.3 MW). For top 6 selected slot champions i in [1..6], compute normalized weight w_i,k = (1 / MAE_bar_i,k) / SUM(1 / MAE_bar_j,k).

Step 5.5 Apply Empirical MOS Linear Calibration Gain: Apply diurnal MOS linear calibration gain to slot blended profile: P_calib_k(b) = a_k * SUM(w_i,k * P_base_i(b)) + b_k.

Step 5.6 Raised-Cosine Slot Cross-Fading: Blend slot boundary transitions (Blocks 39-42 and Blocks 55-58) using 4-block raised-cosine weights w_fade = 0.5 * (1 - cos(pi * idx / 3)) to ensure seamless continuity across slot changes.

Step 5.7 Persistence: Save fused profile to da_mos_consensus_{site_id}_{D_target}.csv. RETURN P_mos_da[1..96].

3.6 Module 6: Planned Hardware Maintenance & Availability Derate Engine

• Algorithm Inputs: P_mos_da[1..96], Maintenance_Log (scheduled outages for Day T+1), P_CAP_AC.

• Algorithm Outputs: P_avail[1..96] (Derated active plant capacity envelope), P_mos_derated[1..96].

Algorithmic Procedure:

Step 6.1 Ingest Day T+1 Maintenance & Outage Schedule: Query plant maintenance registry for target date D_target. Extract planned inverter outages (N_inv_outage(b)), transformer maintenance windows, and tracker array isolation events.

Step 6.2 Calculate Active Interconnect Capacity Envelope: FOR each block b in {1, ..., 96}: compute active available capacity: P_avail(b) = P_CAP_AC * ((N_inv_total - N_inv_outage(b)) / N_inv_total).

Step 6.3 Apply Capacity Ceiling Derate: FOR each block b: set P_mos_derated(b) = min(P_mos_da(b), P_avail(b)).

Step 6.4 Log Maintenance Audit Trail: Record derate details in execution log: log site_id, D_target, affected blocks, and capacity reduction MW.

Step 6.5 Save Availability Profile: Write derate profile to da_capacity_derate_{site_id}_{D_target}.json. RETURN {P_avail, P_mos_derated}.

3.7 Module 7: Master Day-Ahead Prompt Construction & Atmospheric Context Assembly

• Algorithm Inputs: Site_Config, D_target, P_clearsky, P_mos_derated, NWP_Indicators, 24h_MOS_Rankings.

• Algorithm Outputs: Master Day-Ahead Scheduling Prompt text file written to disk.

Algorithmic Procedure:

Step 7.1 Assemble SECTION 1: System & Gate Closure Constraints: Embed AC Interconnect Capacity P_CAP_AC, DC Installed Capacity P_CAP_DC, Transfer Ratio eta_transfer, and statutory 10:00 AM gate closure rules for D_target.

Step 7.2 Assemble SECTION 2: Synoptic Weather Indicators for Target Date: Inject synoptic indicators: convective CAPE (J/kg), total cloud cover %, relative humidity %, AOD, 10m wind speed, and NWP model divergence metric.

Step 7.3 Assemble SECTION 3: 24h Lead-Time MOS Accuracy Context: Embed rolling 7-day 24h lead-time MAE rankings of top ECMWF, ICON, GFS, and GEM candidates across diurnal slots.

Step 7.4 Construct SECTION 4: 96-Block Comparison Data Matrix Table: FOR each block b in {1, ..., 96}: format line [Block b | Time | Status | ClearSky_MW | MOS_Consensus_MW | ECMWF_Top_MW | GFS_Top_MW | ICON_Top_MW].

Step 7.5 Inject SECTION 5: Strictly Enforced JSON Target Schema: Append target JSON schema requiring fields: 'synoptic_regime', 'quantile_bias_factor', 'preferred_nwp_family', 'is_maintenance_outage', 'reasoning', and 'block_predictions' (96-block MW array).

Step 7.6 Save Prompt to Disk: Write prompt to master_da_prompt_{site_id}_{D_target}.txt. RETURN Prompt_Text.

3.8 Module 8: Strategic Day-Ahead LLM Arbiter & Resilient Key Failure Matrix

• Algorithm Inputs: Prompt_Text, D_target, P_CAP_AC, OPENROUTER_API_KEYS pool.

• Algorithm Outputs: Validated Day-Ahead StrategicAdvice JSON payload containing 96-block predictions.

Algorithmic Procedure:

Step 8.1 Initialize Multi-Key Execution Gateway: Configure OpenRouter Gateway endpoint https://openrouter.ai/api/v1/chat/completions. Load 3-key API pool [KEY_PRIMARY, KEY_SECONDARY, KEY_GLOBAL_FAILOVER].

Step 8.2 Invoke Target Foundation Model: POST request with payload: model = 'openai/gpt-5.6-luna', temperature = 0.20, max_tokens = 2500, response_format = {'type': 'json_object'}). Timeout set to 60 seconds.

Step 8.3 Execute Key Rotation & Failover Matrix: IF API returns HTTP 429 (Rate Limit) or HTTP 503 (Unavailable): catch error; execute exponential backoff retry (1s, 2s, 4s); automatically rotate to next key in pool.

Step 8.4 Parse Raw Response & Strip Fences: Receive JSON string response. Automatically strip markdown fences (```json ... ```). Parse JSON via json.loads().

Step 8.5 Validate Schema & Synoptic Regime Positioning: Validate payload schema. Extract synoptic_regime in ('CLEAR_SKY', 'HIGH_CIRRUS', 'MONSOON_OVERCAST', 'LOCAL_CONVECTIVE'). Clamp quantile_bias_factor in [0.70, 1.08].

Step 8.6 Extract 96-Block Direct Predictions: Extract scheduled MW array pred_mw[1..96] from block_predictions. Clamp pred_mw(b) in [0.0, P_avail(b)].

Step 8.7 Deterministic Fail-Safe Fallback SLA: IF key pool exhausted or JSON parse fails: trigger instant fail-safe fallback path; substitute Module 5 MOS consensus profile P_mos_derated[1..96] with 0ms execution SLA.

Step 8.8 Save Payload: Save output to da_llm_schedule_{site_id}_{D_target}.json. RETURN StrategicAdvice.

3.9 Module 9: Physical Guardrail Screening & Final Day-Ahead Schedule Arbitration

• Algorithm Inputs: StrategicAdvice, P_mos_derated[1..96], P_clearsky[1..96], P_avail[1..96], P_CAP_AC.

• Algorithm Outputs: P_da[1..96] (Official 96-block Day-Ahead dispatch schedule vector for Day T+1).

Algorithmic Procedure:

Step 9.1 Enforce GUARDRAIL 1 (Solar Geometry Night Zeroing): FOR each block b in {1, ..., 96}: IF block b < 24 (Time < 05:45 IST) OR block b > 74 (Time >= 18:30 IST): set P_da(b) = 0.00 MW strictly.

Step 9.2 Enforce GUARDRAIL 2 (Clear-Sky Floor Lock): FOR daylight blocks b: IF Kt(b) >= 0.85 OR Regime == 'CLEAR_SKY': IF pred_mw(b) < P_mos_derated(b): set pred_mw(b) = P_mos_derated(b). Protects midday clear-sky output from artificial down-biasing.

Step 9.3 Enforce GUARDRAIL 3 (Overcast Optical Cloud Ceiling): FOR daylight blocks b: IF Kt(b) < 0.50 OR Regime in ('MONSOON_OVERCAST', 'RAIN'): compute overcast ceiling P_max = max(P_mos_derated(b) * 1.10, P_CAP_AC * 0.25). IF pred_mw(b) > P_max: set pred_mw(b) = P_max.

Step 9.4 Enforce GUARDRAIL 4 (Ramp-Rate Continuity & Capacity Clamping): Set maximum allowable 15-minute ramp step P_max_ramp = max(0.50 MW, P_CAP_AC * 0.10). FOR block b from 25 to 74: IF |pred_mw(b) - P_da(b-1)| > P_max_ramp: set pred_mw(b) = P_da(b-1) ± P_max_ramp. Clamp P_da(b) in [0.0, P_avail(b)].

Step 9.5 Compile Final Day-Ahead Schedule Vector: Assemble final 96-block schedule vector P_da[1..96]. Write vector to da_final_schedule_{site_id}_{D_target}.csv.

Step 9.6 Save Schedule Vector: Save vector to disk and return final Day-Ahead vector P_da[1..96].

3.10 Module 10: LLM JSON-to-CSV Conversion, Statutory Submission & S3 Cloud Dispatch

• Algorithm Inputs: P_da[1..96], P_clearsky[1..96], P_mos_derated[1..96], plant_state, S3_BUCKET.

• Algorithm Outputs: Canonical 7-column DA regulatory CSV, Plotly HTML plot, confirmed S3 URIs.

Algorithmic Procedure:

Step 10.1 LLM JSON-to-CSV Conversion Algorithm: Strip markdown fences (```json ... ```) from LLM output string if present. Parse JSON string into dictionary dict_obj = json.loads(clean_str). Extract block_predictions dictionary. Map block values to float MW array pred_mw[1..96]. Enforce 7-column canonical schema: [Block, Time Interval, clearsky_poa_w_m2, mos_consensus_mw, llm_quantile_mw, da_schedule_mw, active_capacity_mw]. Write to da_final_schedule_{site_id}_{D_target}.csv.

Step 10.2 Evaluate Commercial DSM Risk Envelope: Compute expected commercial DSM shortfall risk bandwidth under state regulatory deadbands (TSERC 15%, MPERC 10%, MERC 10%). Confirm schedule positioning minimizes expected penalty loss.

Step 10.3 Render Interactive Day-Ahead Evaluation Graphic: Initialize Plotly figure go.Figure(). Add Trace 1: Clear-Sky Physical Ceiling (amber dashed line); Trace 2: MOS Consensus Forecast (green dotted line); Trace 3: Strategic LLM Raw Prediction (purple line); Trace 4: Official Arbitrated DA Schedule (solid navy line).

Step 10.4 Export HTML Plot File: Save graphic to da_evaluation_{site_id}_{D_target}.html.

Step 10.5 Enterprise AWS S3 Cloud Synchronization: Upload generated artifacts via boto3 S3 client to s3://{S3_BUCKET}/raw/vedanjay/{site_id}/{D_target}/day_ahead/ before 10:00 AM statutory gate closure.

Step 10.6 Execution Handoff: RETURN {DA_CSV_Path, Plotly_HTML_Path, S3_URI_List}.

4. Data Flow and File Artifacts

The following table summarizes the input data sources, core computational functions, and resulting output file artifacts produced across each execution stage of the Day-Ahead Solar System:

| Module | Input Source | Core Computational Function | Output File Artifact / S3 Destination |
| --- | --- | --- | --- |
| Module 1 (Dynamic Ingestion) | Asia/Kolkata Time & Open-Meteo | Dynamic IST date resolution (D_target = D_today + 1) & 00Z/06Z NWP fetch | da_raw_nwp_feed_{SITE}_{D_TARGET}.json |
| Module 2 (Clear-Sky) | Plant GPS, Tilt, Azimuth | Ineichen clear-sky ephemeris & Perez POA transposition for target D_target | da_clearsky_poa_{SITE}_{D_TARGET}.csv |
| Module 3 (Kt Spline) | Open-Meteo 143 Members | Hourly Kt calculation & physics-guided GTI spline reconstruction | da_gti_matrix_{SITE}_{D_TARGET}.csv |
| Module 4 (Physics Base) | GTI Matrix & Temperature | Sandia cell temperature & PVUSA derated AC base power matrix | da_physics_base_power_{SITE}_{D_TARGET}.csv |
| Module 5 (Top-6 MOS) | P_base Matrix & SCADA 7D | 24h lead-time MAE ranking, Agency Diversity Cap (max 2/agency), Bayesian weight | da_mos_consensus_{SITE}_{D_TARGET}.csv |
| Module 6 (Maintenance) | Plant Outage Logs & MOS | Scheduled inverter outage derating & active capacity ceiling | da_capacity_derate_{SITE}_{D_TARGET}.json |
| Module 7 (Master Prompt) | MOS Forecasts & Synoptic | Synoptic indicators injection & Day-Ahead prompt matrix construction | master_da_prompt_{SITE}_{D_TARGET}.txt |
| Module 8 (DA LLM Engine) | Master Prompt & OpenRouter | OpenRouter GPT-5.6 Luna inference & 3-key failover matrix | da_llm_schedule_{SITE}_{D_TARGET}.json |
| Module 9 (Guardrails) | LLM Output & Physics | 4 physical guardrail filters & Day-Ahead schedule arbitration | da_final_schedule_{SITE}_{D_TARGET}.csv |
| Module 10 (Submit & S3) | Final DA Schedule | Canonical 7-column DA CSV schema, Plotly HTML, S3 push by 10 AM | da_evaluation_{SITE}_{D_TARGET}.html |