Intellis Solar Power Generation Scheduling System
Low-Level Algorithmic Specification, Solar Radiative Physics, and Strategic LLM Pipeline

1. Global Variables and System Parameters

This section documents the primary operational parameters, mathematical constants, astronomical thresholds, clear-sky irradiance parameters, and grid timing constraints governing the Intellis Solar Power Scheduling System. All values represent standardized physical properties and operational bounds enforced across all solar utility-scale assets.

| Parameter Name | Symbol / Variable | Default Value / Bound | System Description & Role |
| --- | --- | --- | --- |
| AC Interconnect Capacity | P_CAP_AC | 5.0 - 50.0 MW | Maximum rated active power export capacity at grid interconnection point. |
| DC Installed Capacity | P_CAP_DC | 6.5 - 65.0 MWp | Total installed peak DC capacity of solar PV array under Standard Test Conditions (STC). |
| DC-to-AC Transfer Ratio | eta_transfer | 0.75 - 0.85 | Physical transfer ratio converting DC array output to AC grid export power. |
| Site Barometric Altitude | z | 250.0 - 650.0 m | Site elevation above mean sea level used for barometric pressure and air mass calculation. |
| Panel Tilt Angle | beta | 15.0° - 24.0° | Fixed tilt angle of solar PV module array relative to horizontal plane. |
| Panel Azimuth Angle | gamma_m | 180.0° (South) | Orientation angle of PV array relative to True North (180° = South facing). |
| Linke Turbidity Factor | T_L | 3.0 - 4.5 | Atmospheric aerosol and water vapor attenuation factor in Ineichen clear-sky model. |
| Temperature Coefficient | gamma_Pmp | -0.37% / °C | Temperature derating coefficient of maximum power for PV modules. |
| Faiman Convection Coeff | u0, u1 | u0=29.0, u1=0.0 | Sandia Faiman thermal model constants converting GTI and wind speed to cell temperature. |
| Clear-Sky Clearness Index | Kt_clear | 0.85 | Threshold clearness ratio above which midday atmosphere is classified as Clear-Sky. |
| Overcast Clearness Index | Kt_overcast | 0.50 | Threshold clearness ratio below which atmosphere is classified as Overcast / Optical Cloud. |
| Zenith Angle Cutoff | theta_z_cutoff | 87.0° (elevation < 3°) | Solar zenith angle cutoff below which clear-sky POA irradiance is clamped strictly to 0.0 W/m². |
| Regulatory Freeze Lag | T_freeze_lag | 45 min / 90 min | Grid operator gate-closure lag (3 blocks for TSERC/MERC, 6 blocks for MPERC). |
| Actionable Horizon Length | H | 12 - 96 blocks | Number of forward 15-minute grid blocks optimized during active scheduling revision. |
| Telemetry PR Decay Constant | tau_PR | 8.0 blocks | Exponential decay lookback parameter for dynamic Performance Ratio momentum blending. |
| Momentum Nudge Vector | w_momentum | [0.70, 0.45, 0.20] | 3-block weights applied to SCADA actuals for immediate post-freeze horizon blocks. |
| LLM Inference Temperature | T_llm | 0.15 - 0.20 | Sampling temperature for OpenRouter GPT-5.6 Luna to ensure deterministic reasoning. |
| LLM Foundation Model | Model_LLM | openai/gpt-5.6-luna | Target LLM strategic arbiter invoked via OpenRouter API Gateway. |

2. End-to-End System Execution Flow

At each scheduled 15-minute grid revision gate R in {1, 2, ..., 96}, the Intellis Solar Power Scheduling System executes a rigorous 10-step sequential pipeline to produce the official 96-block generation schedule deliverable to state utility load dispatch centers (SLDCs):

Step 1: Telemetry Data Ingestion, Resampling, & Satellite Bypass: Retrieve SCADA plant telemetry for historical blocks up to R-1 from AWS S3 data lakes; clean non-numeric (#N/A) noise; resample to 15-minute grid timestamps; aggregate active inverter power; apply satellite bypass routing for remote unmetered plants.

Step 2: Clear-Sky Astronomical Solar Modeling: Calculate solar ephemeris (zenith, azimuth, air mass) and evaluate Ineichen clear-sky radiation triplet (GHI, DNI, DHI); apply Perez/Hay-Davies POA transposition to establish deterministic theoretical clear-sky physical ceiling envelope POA_cs(b).

Step 3: Multi-Agency NWP Super-Ensemble & Physics-Guided Kt Spline: Fetch 143 ensemble weather forecast members across GFS, ECMWF IFS, ICON, and GEM via Open-Meteo API; compute hourly clearness ratio Kt(H) = GHI_fcst / GHI_cs; resample Kt to 15-minute midpoints via cubic spline; reconstruct high-resolution GTI_m(b) preserving Earth rotation curvature.

Step 4: Faiman/Sandia Thermal Convection & Physics Base Generation: Simulate PV module cell temperature T_cell_m(b) using Sandia thermal convection model; derate DC output using temperature coefficient gamma_Pmp; pass through inverter conversion efficiency and site transfer multipliers to compute 143-member physics base power array P_base_m(b).

Step 5: 3-Slot Diurnal Portfolio Selection & Inverse-Loss Weighting: Partition solar day into 3 independent time slots (Morning, Midday, Evening); evaluate candidate historical loss; enforce agency diversity capping (max 2 models per family); select slot champions; compute Bayesian inverse-loss weights and apply MOS linear calibration gain.

Step 6: Real-Time SCADA Telemetry Ingestion & Intraday Horizon Bridge: Extract live meter telemetry up to block R-1; construct 30m meter / 45m weather horizon bridge (100% meter for Δb in {0,1}, 50% meter / 50% weather bridge for Δb=2, 100% weather ensemble for Δb >= 3); zero out night blocks (b < 24 or b > 74).

Step 7: Master Prompt Construction & Layer 2 Diagnostic Assembly: Resolve state regulatory freeze lag D (3 blocks for TSERC/MERC, 6 blocks for MPERC); segment horizon into frozen buffer [R .. R+D-1] and live target window [R+D .. R+D+11]; inject Layer 2 feedback directives, weather indicators, and multi-model forecast matrix into structured master prompt.

Step 8: OpenRouter Strategic LLM Arbiter & Resilient Key Failure Matrix: Invoke OpenRouter API Gateway targeting openai/gpt-5.6-luna with fallback across 3-key rotation pool; parse JSON payload; extract atmospheric regime classification, quantile bias factor, and direct 12-block predictions; fallback to pure physics on API failure.

Step 9: Four Physical Guardrail Screening & Final Schedule Arbitration: Screen raw predictions against Guardrail 1 (Solar geometry zeroing), Guardrail 2 (Clear-sky floor lock), Guardrail 3 (Overcast optical cloud ceiling), and Guardrail 4 (Ramp-rate continuity filter); stitch frozen past, LLM live window, and ensemble afternoon tail into final schedule vector P_schedule.

Step 10: Multi-State Regulatory Penalty Accounting & AWS S3 Sync: Evaluate block-by-block DSM deviation penalties under state-specific regulatory deadbands (TSERC 15%, MPERC 10%, MERC 10%); render Plotly HTML interactive evaluation graphics; push canonical 7-column CSV artifacts to AWS S3 enterprise data lake.

3. Module-by-Module Algorithmic Logic

This section details the exact low-level algorithmic logic, mathematical equations, data input structures, output specifications, and step-by-step procedures for each of the 10 computational modules comprising the Intellis Solar System.

3.1 Module 1: Telemetry Data Ingestion, Resampling, and Hardware Profiling

• Algorithm Inputs: site_id, run_date, revision_block R, P_CAP_AC, P_CAP_DC, S3_BUCKET, lookback_days = 3.

• Algorithm Outputs: DF_clean containing audited columns [timestamp, block, power_mw, poa_raw, temp_cell] for 96 grid blocks.

Algorithmic Procedure:

Step 1.1 Check Satellite Bypass Classification: IF site_id is in SATELLITE_DERIVED_SITES (e.g. CME, CHAKUR, GUGGI): bypass SCADA hardware meter pipeline; route execution directly to Satellite Irradiance Engine; set IS_SATELLITE_MODE = True.

Step 1.2 Initialize Telemetry Retrieval Window: Initialize empty list Day_Frames = []. FOR offset_day in {3, 2, 1, 0}: set target_date = run_date - offset_day. Query AWS S3 path s3://{S3_BUCKET}/raw/vedanjay/{site_id}/{target_date}/scada_actuals.csv.

Step 1.3 Timestamp Normalization & Data Cleansing: Parse timestamp strings to datetime in timezone Asia/Kolkata. Convert active power column using pd.to_numeric(errors='coerce'). FOR each row i: IF power_raw is non-numeric, blank, '#N/A', NULL, or NaN: preserve NaN flag to avoid skewing zero-generation statistics.

Step 1.4 15-Minute Grid Discretization: Map timestamp to 15-minute grid block index b in {1, 2, ..., 96}: block = floor((hour * 60 + minute) / 15) + 1.

Step 1.5 Inverter Output Aggregation: IF plant contains multiple inverter stations (N_inv > 1): aggregate total active power export P_meter(b) = SUM(inverter_kw_k(b)) / 1000.0. Validate individual inverter status flags to log partial array tripping.

Step 1.6 Night Clamping & Physical Boundary Validation: FOR each block b in {1, ..., 96}: IF block < 24 (Time < 05:45 IST) OR block > 74 (Time >= 18:30 IST): set power_mw(b) = 0.00 MW strictly. ELSE: set power_mw(b) = clip(power_mw(b), 0.0, P_CAP_AC).

Step 1.7 Data Persistence: Save processed DataFrame to metered_actuals_{run_date}.csv in local runtime directory. RETURN DF_clean[1..96].

3.2 Module 2: Ineichen Clear-Sky Solar Ephemeris & Transposition Engine

• Algorithm Inputs: latitude, longitude, altitude z, tilt beta, azimuth gamma_m, Linke turbidity T_L, run_date.

• Algorithm Outputs: POA_cs[1..96] (Clear-Sky Plane-of-Array Irradiance array in W/m²).

Algorithmic Procedure:

Step 2.1 Calculate High-Precision Solar Ephemeris: FOR each 15-minute midpoint t_b (b = 1..96): evaluate solar zenith angle theta_z(b), solar azimuth angle gamma_s(b), and solar elevation angle alpha(b) = 90° - theta_z(b) using NREL SPA / PVLib solar position algorithm.

Step 2.2 Evaluate Ineichen Clear-Sky Radiation Triplet: Compute relative air mass AM(b) with barometric altitude correction AM_corr = AM * (P_baro / 101325). Compute Ineichen Clear-Sky Irradiance Triplet: Direct Normal DNI_cs(b), Diffuse Horizontal DHI_cs(b), Global Horizontal GHI_cs(b) = DNI_cs(b) * cos(theta_z(b)) + DHI_cs(b).

Step 2.3 Compute Angle of Incidence (AOI): Calculate AOI theta(b) on tilted PV array: cos(theta) = cos(theta_z) * cos(beta) + sin(theta_z) * sin(beta) * cos(gamma_s - gamma_m).

Step 2.4 Hay-Davies / Perez Plane-of-Array Transposition: Decompose clear-sky POA into 3 components: Direct Beam POA_beam = DNI_cs * max(0, cos(theta)); Ground Albedo Reflection POA_ground = GHI_cs * 0.20 * (1 - cos(beta)) / 2; Sky Diffuse POA_diffuse via Hay-Davies anisotropic model. Total POA_cs(b) = POA_beam + POA_ground + POA_diffuse.

Step 2.5 Elevation Cutoff & Horizon Clamping: IF solar elevation alpha(b) < 3.0° (or theta_z(b) >= 87.0°): clamp POA_cs(b) = 0.00 W/m² to prevent atmospheric edge refraction noise.

Step 2.6 Persistence & Export: Save theoretical profile to clear_sky_power_forecast_{run_date}.csv. RETURN POA_cs[1..96].

3.3 Module 3: 143-Member NWP Super-Ensemble & Physics-Guided Kt Spline Engine

• Algorithm Inputs: latitude, longitude, run_date, target_date_str, Open-Meteo High-Bandwidth Endpoint.

• Algorithm Outputs: GTI_matrix[143][96] (Discretized 15-minute Plane-of-Array Irradiance matrix for all 143 NWP members).

Algorithmic Procedure:

Step 3.1 Fetch 143-Member Multi-Agency Super-Ensemble Feed: Query Open-Meteo Ensemble API across 4 NWP agency families (ECMWF IFS 35 members, ICON-EU/Global 40 members, GFS/GEFS 30 members, GEM 38 members). Extract hourly irradiance variables GHI_m(H), DNI_m(H), DHI_m(H) for H in {0, ..., 23} for all members m in {1, ..., 143}.

Step 3.2 Address Direct Linear Interpolation Failure: Direct linear interpolation of hourly irradiance creates severe unphysical trapezoidal step discontinuities near sunrise and sunset. Enforce Physics-Guided Clearness Ratio Kt Spline Reconstruction.

Step 3.3 Compute Hourly Clearness Ratio Kt(H): FOR each member m and each hour H in {0, ..., 23}: compute hourly clearness index Kt_m(H) = GHI_m(H) / max(1.0, GHI_cs(H)). Clamp Kt_m(H) in [0.0, 1.20].

Step 3.4 Cubic Spline Interpolation of Kt to 15-Min Midpoints: Fit natural cubic spline over 24 hourly values Kt_m(H); evaluate spline at 96 15-minute block midpoints to produce continuous clearness profile Kt_m(b) for b in {1, ..., 96}.

Step 3.5 Physics-Guided Reconstruction of 15-Min GTI Matrix: Multiply continuous clearness profile by high-resolution instantaneous clear-sky POA: GTI_m(b) = Kt_m(b) * POA_cs(b). This perfectly recovers natural Earth rotation curvature while preserving NWP cloud attenuation.

Step 3.6 Save Irradiance Matrix: Save candidate array to openmeteo_gti_matrix_{run_date}_block_{R}.csv. RETURN GTI_matrix[143][96].

3.4 Module 4: Faiman/Sandia Convective Thermal Model & Physics Base Generation

• Algorithm Inputs: GTI_matrix[143][96], T_air_m(b), wind_10m_m(b), P_CAP_DC, P_CAP_AC, gamma_Pmp, u0, u1, M[b].

• Algorithm Outputs: P_base_matrix[143][96] (Thermally derated AC power matrix for all 143 candidates).

Algorithmic Procedure:

Step 4.1 Evaluate Sandia Cell Temperature: FOR each member m and each block b: evaluate PV cell temperature using Sandia Faiman model: T_cell_m(b) = T_air_m(b) + GTI_m(b) * exp(-u0 - u1 * wind_10m_m(b)).

Step 4.2 Compute Thermal Power Derating Factor: Compute module temperature efficiency factor: derate_temp_m(b) = 1.0 + (gamma_Pmp / 100.0) * (T_cell_m(b) - 25.0).

Step 4.3 Simulate DC & AC Generation Profiles: Compute raw DC power: P_dc_m(b) = P_CAP_DC * (GTI_m(b) / 1000.0) * derate_temp_m(b). Convert to AC power via inverter efficiency curve: P_ac_m(b) = min(P_dc_m(b) * eta_inverter, P_CAP_AC).

Step 4.4 Apply Site Empirical Array Transfer Multipliers: Multiply AC output by 96-block calibrated array transfer weights: P_base_m(b) = round(P_ac_m(b) * M[b], 2).

Step 4.5 Enforce Clear-Sky Physical Ceiling: Clamp base power strictly below clear-sky envelope: P_base_m(b) = min(P_base_m(b), P_clearsky(b)).

Step 4.6 Save Candidate Base Power: Write candidate matrix to physics_base_power_{run_date}_block_{R}.csv. RETURN P_base_matrix[143][96].

3.5 Module 5: 3-Slot Diurnal Portfolio Selection & Inverse-Loss Weighting Engine

• Algorithm Inputs: P_base_matrix[143][96], historical SCADA actuals A[b], rolling lookback window.

• Algorithm Outputs: GTI_fused[1..96], P_base_fused[1..96] (Fused diurnal ensemble profiles).

Algorithmic Procedure:

Step 5.1 Partition Solar Day into 3 Diurnal Slots: Divide 96 blocks into 3 operational slots: Slot 1 (Morning: Blocks 24-40 / 06:00-10:00 IST), Slot 2 (Midday: Blocks 41-56 / 10:00-14:00 IST), Slot 3 (Evening: Blocks 57-74 / 14:00-18:30 IST).

Step 5.2 Calculate Historical MAE / Loss per Candidate: FOR each member m in {1, ..., 143}: evaluate Mean Absolute Error over rolling 3-day lookback window in each slot k: MAE_m,k = mean(|P_base_m(b) - A(b)|) for b in Slot_k.

Step 5.3 Enforce Agency Diversity Capping Algorithm: Rank candidates in each slot by ascending MAE. Select top candidates while enforcing strict Agency Diversity Cap: max 2 members per NWP family (ECMWF, ICON, GFS, GEM) to prevent model redundancy.

Step 5.4 Compute Bayesian Inverse-Loss Weights: For the top 6 selected slot champions m = 1..6: compute normalized inverse-loss weight: w_m,k = (1.0 / MAE_m,k) / SUM(1.0 / MAE_j,k).

Step 5.5 Apply Empirical MOS Linear Calibration Gain: Apply diurnal MOS linear regression calibration gain to slot blended profile: P_calib_k(b) = a_k * SUM(w_m,k * P_base_m(b)) + b_k.

Step 5.6 Raised-Cosine Slot Cross-Fading: Blend overlapping slot boundaries using 4-block raised-cosine transition weights w_fade = 0.5 * (1 - cos(pi * idx / 4)) to ensure seamless continuity across slot transitions.

Step 5.7 Save Fused Profile: Save consensus forecast to fused_diurnal_ensemble_{run_date}_block_{R}.csv. RETURN P_base_fused[1..96].

3.6 Module 6: Real-Time SCADA Telemetry Ingestion & Intraday Horizon Bridge

• Algorithm Inputs: DF_clean, P_base_fused[1..96], revision_block R, freeze_lag D.

• Algorithm Outputs: P_live_bridge[1..96] (Telemetry-adjusted short-term dispatch baseline).

Algorithmic Procedure:

Step 6.1 Identify Actionable Revision Block: Set actionable starting block B_act = R + D. Extract live meter actuals A[b] for completed blocks b < R.

Step 6.2 Compute Telemetry Performance Ratio (PR): FOR completed daylight blocks b in {max(24, R-8) .. R-1}: compute instantaneous PR(b) = A(b) / max(0.01, P_clearsky(b)). Compute recency-weighted mean PR: PR_recent = SUM(PR(b) * exp(0.4 * b)) / SUM(exp(0.4 * b)).

Step 6.3 Construct 30m Meter / 45m Weather Horizon Bridge: FOR forward blocks Δb = b - R + 1: IF Δb in {0, 1} (0 to 30 min): set P_bridge(b) = A(R-1) (100% SCADA meter tracking to capture immediate outages). IF Δb == 2 (30 to 45 min): compute 50% meter / 50% weather bridge: Kt_eff = 0.50 * Kt_obs + 0.50 * Kt_fcst; P_bridge(b) = Kt_eff * P_clearsky(b). IF Δb >= 3 (Beyond 45 min): set P_bridge(b) = P_base_fused(b) (100% pure weather ensemble).

Step 6.4 Eliminate Legacy Midday Traps: Remove artificial midday suppression clamps; govern afternoon solar recovery strictly via atmospheric radiative transfer equations.

Step 6.5 Apply Hard Night Zeroing: IF block b < 24 OR block b > 74: set P_bridge(b) = 0.00 MW strictly.

Step 6.6 Persistence: Save bridge array to intraday_horizon_bridge_{run_date}_block_{R}.csv. RETURN P_live_bridge[1..96].

3.7 Module 7: Master Prompt Construction & Strategic Context Assembly

• Algorithm Inputs: site_config, run_date, revision_block R, P_clearsky, P_live_bridge, P_base_fused, Directives.

• Algorithm Outputs: Master Scheduling Prompt text file written to disk.

Algorithmic Procedure:

Step 7.1 Determine State Freeze Lag D: IF plant state is 'MADHYA PRADESH': set freeze lag D = 6 blocks (90 min). ELSE (Telangana / Maharashtra): set freeze lag D = 3 blocks (45 min).

Step 7.2 Compute Actionable Horizon Boundaries: Set B_start = R, B_live = R + D, B_end = min(R + D + 11, 96). Segment prompt into Frozen Buffer Zone [R .. B_live-1] and Target Live Window [B_live .. B_end].

Step 7.3 Build SECTION 1: System & Inverter Capacity Constraints: Embed AC Interconnect Capacity P_CAP_AC, DC Installed Capacity P_CAP_DC, Transfer Ratio eta_transfer, and active inverter status.

Step 7.4 Build SECTION 1B: Grid Regulatory Timing & Buffer Zone: Specify frozen block states, gate-closure lead times, and mandatory non-revision horizon parameters.

Step 7.5 Inject SECTION 2 & 3: Role Directives & Atmospheric Summaries: Inject solar atmospheric classification (CLEAR_SKY, PARTLY_CLOUDY, OVERCAST, RAIN), clearness index Kt, cloud cover %, and aerosol optical depth.

Step 7.6 Inject SECTION 4: Layer 2 Diagnostic Performance Feedback: Embed historical MAE/RMSE metrics, outage audit tables, and LLM diagnostic advisory directives from completed blocks.

Step 7.7 Construct SECTION 5: Multi-Model Forecast Comparison Table: FOR each block b in {B_start .. B_end}: format line [Block b | Time | Status | ClearSky_MW | Telemetry_Bridge_MW | Ensemble_MW].

Step 7.8 Inject SECTION 6: Strictly Enforced JSON Schema: Append exact target JSON schema instructions specifying fields 'regime', 'quantile_bias_factor', 'preferred_agency', 'is_trip_or_curtailment', 'reasoning', and 'block_predictions'.

Step 7.9 Save Prompt to Disk: Write master prompt to master_prompt_{site_id}_{run_date}_block_{R}.txt. RETURN Prompt_Text.

3.8 Module 8: OpenRouter LLM Strategic Arbiter & Resilient Key Failure Matrix

• Algorithm Inputs: Prompt_Text, R, B_live, P_CAP_AC, OPENROUTER_API_KEYS pool.

• Algorithm Outputs: Validated StrategicAdvice JSON object containing 12-block predictions.

Algorithmic Procedure:

Step 8.1 Initialize Resilient Multi-Key Execution Gateway: Configure OpenRouter API Gateway endpoint https://openrouter.ai/api/v1/chat/completions. Load API key pool [KEY_PRIMARY, KEY_SECONDARY, KEY_GLOBAL_FAILOVER].

Step 8.2 Invoke Target Foundation Model: POST request with payload: model = 'openai/gpt-5.6-luna', temperature = 0.20, max_tokens = 1500, response_format = {'type': 'json_object'}. Timeout set to 45 seconds.

Step 8.3 Handle Rate Limits & Failover Matrix: IF API returns HTTP 429 (Rate Limit) or HTTP 503 (Service Unavailable): catch error; execute exponential backoff retry; automatically rotate to next key in pool. Max retry attempts = 3.

Step 8.4 Parse Raw Response & Strip Markdown Fences: Receive JSON string response. Automatically strip markdown fences (```json ... ```) if present. Parse JSON payload via json.loads().

Step 8.5 Validate Schema & Enforce Safety Lockouts: Validate payload schema. Enforce No-Trip Hard Lockout: set advice.is_trip_or_curtailment = False (Electrical inverter trips are governed deterministically outside LLM). Clamp quantile_bias_factor: Clear-Sky in [1.00, 1.08], Overcast in [0.60, 1.08].

Step 8.6 Extract 12-Block Strategic Predictions: FOR each target block b in {B_live .. B_live+11}: extract scheduled MW pred_mw(b) from block_predictions. Clamp pred_mw(b) in [0.0, P_CAP_AC].

Step 8.7 Deterministic Fail-Safe Fallback SLA: IF key pool is exhausted OR JSON parsing fails: trigger instant fail-safe fallback path; substitute pure physics base profile P_base_fused(b) with 0ms execution SLA.

Step 8.8 Save Payload: Save output to llm_schedule_{run_date}_block_{R}.json. RETURN StrategicAdvice.

3.9 Module 9: Physical Guardrail Screening & Final Schedule Arbitration

• Algorithm Inputs: StrategicAdvice, P_base_fused[1..96], P_clearsky[1..96], R, B_live, P_CAP_AC.

• Algorithm Outputs: P_schedule[1..96] (Immutable 96-block official dispatch schedule).

Algorithmic Procedure:

Step 9.1 Enforce GUARDRAIL 1 (Solar Geometry Hard Cutoff): FOR each block b in {1, ..., 96}: IF block b < 24 (Time < 05:45 IST) OR block b > 74 (Time >= 18:30 IST): set P_schedule(b) = 0.00 MW strictly. Override any positive LLM prediction during night hours.

Step 9.2 Enforce GUARDRAIL 2 (Clear-Sky Negative Cut Lockout): FOR live blocks b in target window: IF Kt(b) >= 0.85 OR Regime == 'CLEAR_SKY' OR Cloud Cover <= 30%: IF pred_mw(b) < P_base_fused(b): set pred_mw(b) = P_base_fused(b). Protects midday clear-sky output from artificial down-biasing.

Step 9.3 Enforce GUARDRAIL 3 (Overcast Optical Cloud Ceiling): FOR live blocks b in target window: IF Kt(b) < 0.50 OR Regime in ('OVERCAST', 'RAIN') OR Cloud Cover >= 75%: compute overcast ceiling P_max = max(P_base_fused(b) * 1.10, P_CAP_AC * 0.25). IF pred_mw(b) > P_max: set pred_mw(b) = P_max.

Step 9.4 Enforce GUARDRAIL 4 (Ramp-Rate Continuity Filter): Set maximum allowable 15-minute ramp step: P_max_ramp = max(0.50 MW, P_CAP_AC * 0.10). FOR block b from B_live to 96: IF |pred_mw(b) - P_schedule(b-1)| > P_max_ramp: set pred_mw(b) = P_schedule(b-1) ± P_max_ramp.

Step 9.5 Perform Hybrid Schedule Assembly & Stitching: Assemble final schedule vector P_schedule[1..96]: Blocks 1 to B_live-1 preserved from frozen past schedule; Blocks B_live to B_live+11 set to Guardrail-screened LLM predictions; Blocks B_live+12 to 96 set to thermally derated ensemble tail.

Step 9.6 Save Immutable Schedule Vector: Write arbitrated schedule to strategy2_frozen_forecast_{site_id}_{run_date}.csv. RETURN P_schedule[1..96].

3.10 Module 10: Multi-State Regulatory Penalty Engine & Cloud Asset Synchronization

• Algorithm Inputs: P_schedule[1..96], SCADA actuals A[1..96], P_CAP_AC, plant_state, S3_BUCKET.

• Algorithm Outputs: Penalty summary report, Plotly interactive HTML plot, and confirmed S3 upload URIs.

Algorithmic Procedure:

Step 10.1 Resolve State Regulatory Parameters: IF plant_state is 'TELANGANA' (TSERC): Deadband = 15.0%. Tier 1 (15-25%, ₹0.50/kWh), Tier 2 (25-35%, ₹1.00/kWh), Tier 3 (>35%, ₹1.50/kWh). IF plant_state is 'MADHYA PRADESH' (MPERC): Deadband = 10.0%. Tier 1 (10-15%, ₹0.50/kWh), Tier 2 (15-20%, ₹0.75/kWh), Tier 3 (>20%, ₹1.00/kWh). IF plant_state is 'MAHARASHTRA' (MERC): Deadband = 10.0%. Tier 1 (10-12%, ₹0.25/kWh), Tier 2 (12-15%, ₹0.50/kWh), Tier 3 (15-25%, ₹0.75/kWh), Tier 4 (>25%, ₹1.00/kWh).

Step 10.2 Compute Block-by-Block Commercial Penalties: FOR each block b in {1, ..., 96}: IF A(b) is NaN (Missing SCADA): Penalty(b) = ₹0.00. ELSE: compute absolute deviation E(b) = |A(b) - P_schedule(b)| and percentage delta(b) = (E(b) / P_CAP_AC) * 100.0. IF delta(b) <= Deadband: Penalty(b) = ₹0.00. ELSE: apply tiered commercial pricing on excess deviation slice.

Step 10.3 Render Interactive Plotly Visualization Graphic: Initialize Plotly figure go.Figure(). Add Trace 1: SCADA Actual Power (bold black line); Trace 2: Regulatory Allowance Envelope (gray band); Trace 3: Clear-Sky Ceiling (amber dashed line); Trace 4: Multi-Model Ensemble (green dotted line); Trace 5: Official Frozen Schedule (solid crimson line). Attach interactive range filter buttons.

Step 10.4 Export HTML Plot File: Save graphic to strategy2_evaluation_{site_id}_{run_date}.html.

Step 10.5 Enterprise AWS S3 Data Sync: FOR each output artifact file (CSV, JSON, HTML): upload file to target S3 path s3://{S3_BUCKET}/raw/vedanjay/{site_id}/{run_date}/forecasts/ using boto3 S3 client.

Step 10.6 Execution Handoff: RETURN Total_Daily_Penalty_INR, S3_URI_List.

4. Data Flow and File Artifacts

The following table summarizes the input data sources, core computational functions, and resulting output file artifacts produced across each execution stage of the Intellis Solar System:

| Module | Input Source | Core Computational Function | Output File Artifact / S3 Destination |
| --- | --- | --- | --- |
| Module 1 (Telemetry) | S3 SCADA Actuals | Inverter aggregation, #N/A preservation, satellite bypass | metered_actuals_{DATE}.csv |
| Module 2 (Clear-Sky) | Plant GPS, Tilt, Azimuth | Ineichen clear-sky ephemeris & Perez POA transposition | clear_sky_power_forecast_{DATE}.csv |
| Module 3 (Kt Spline) | Open-Meteo 143 Members | Hourly Kt calculation & physics-guided GTI spline | openmeteo_gti_matrix_{DATE}_block_{R}.csv |
| Module 4 (Physics Base) | GTI Matrix & Temperature | Sandia cell temperature & PVUSA derated AC power | physics_base_power_{DATE}_block_{R}.csv |
| Module 5 (Portfolio) | P_base Matrix & SCADA | 3-slot diurnal ranking, agency capping, inverse-loss weight | fused_diurnal_ensemble_{DATE}_block_{R}.csv |
| Module 6 (Horizon Bridge) | SCADA Actuals & Fused | 30m meter / 45m weather horizon bridge & night clamp | intraday_horizon_bridge_{DATE}_block_{R}.csv |
| Module 7 (Master Prompt) | Model Forecasts & Site Config | Freeze buffer segmentation & Layer 2 prompt construction | master_prompt_{SITE}_{DATE}_block_{R}.txt |
| Module 8 (LLM Engine) | Master Prompt | OpenRouter GPT-5.6 Luna inference & 3-key failover matrix | llm_schedule_{DATE}_block_{R}.json |
| Module 9 (Guardrails) | LLM Output & Physics | 4 physical guardrail filters & cumulative frozen stitching | strategy2_frozen_forecast_{SITE}_{DATE}.csv |
| Module 10 (Penalty & S3) | Frozen Schedule & Actuals | Multi-state SERC DSM penalty audit, Plotly HTML, S3 push | strategy2_evaluation_{SITE}_{DATE}.html |